# PROJECT READINESS NOTE

Team ID : P124

Project Title: ML-Assisted Simulated Annealing for RTL-Accelerated VLSI Macro Placement
Domain/Area: Machine Learning, VLSI Physical Design, EDA Tool Development.
Team Members:
- K Sahana

- Ratik Agrawal
- Neha Ramiah
Team Leader: K Sahana
Mentor Name: Dr. Krupa Rasane

Problem Understanding

Background of the Problem: During the physical design phase of VLSI development, macro placement is an NP-hard optimization problem heavily bottlenecked by sequential CPU calculations. Suboptimal placement degrades a chip’s Power, Performance, and Area (PPA) metrics.

Importance and Real-World Relevance: Modern SoC design requires extremely fast placement turnaround times. Existing heuristic algorithms (like Simulated Annealing) achieve high accuracy but are severely bottlenecked by slow software execution.

Existing Solutions and Limitations: Pure Machine Learning placement models generate coordinates rapidly but fundamentally fail to enforce strict zero-overlap physical design rules, resulting in illegal, unmanufacturable layouts.

Gap we aim to address: There is a critical need for a drop-in heterogeneous accelerator. This project aims to hijack the standard EDA flow, replacing the traditional software macro placer with a high-speed ML Predictor coupled with a deterministic, zero-overlap RTL Hardware Legalizer.

# Proposed Methodology & Tools

Proposed Approach: We are building a custom Hardware-Software Co-Design point tool that integrates directly into the open-source OpenROAD physical design flow. OpenROAD handles synthesis and floorplanning, then halts. A Python orchestrator extracts the data, feeds it to an ML Predictor for an approximate layout, and then passes it to a Verilog RTL state machine that strictly legalizes it. The final layout is injected back into OpenROAD for routing, native GUI visualization, and Static Timing Analysis (STA) signoff.

Architecture / Workflow:

- 1. Automated Orchestration: A Python script utilizing the subprocess module acts as the master controller, providing a single-click automated execution of the entire pipeline.
2. The Extraction (OpenROAD): OpenROAD synthesizes a netlist and halts, dumping the chip's physical state to a Design Exchange Format (.def) file. Macro orientations are strictly locked to default (N) to prevent routing failure.
3. The ML Predictor (Python): Python parses the .def file. A Supervised ML model predicts approximate, wirelength-optimized coordinates (with expected overlaps) and formats them into a .hex memory file.
4. The RTL Legalizer (Verilog): A custom Verilog Simulated Annealing engine reads the .hex file. Operating at a low initialization temperature with a massive overlap penalty, it uses parallel combinational logic to physically push overlapping blocks apart. Upon completion, a Verilog testbench automatically exports the legalized coordinates to a new output file via $fdisplay / $writememh.
5. The Injection & Signoff (OpenROAD): Python rewrites the final Verilog coordinates back into the .def file. OpenROAD reads the modified file, renders the legalized floorplan natively in the OpenROAD GUI, and resumes execution to run routing and STA signoff.

Tools & Technologies:

- Programming languages: Python 3.x, Verilog HDL, TCL.

- Frameworks/Suites: OpenROAD (Physical Design EDA Suite), Scikit-Learn / PyTorch (ML Predictor).

- Platforms: Icarus Verilog (iverilog) for RTL simulation, Git/GitHub for version control.

# Initial References & Datasets

A. Key References (3–5):

1.  Title: Hardware Acceleration of Simulated Annealing for Constraint Satisfaction Problems

- Authors: S. Das, A. Pannone, et al.

- Year: 2025

- Source: Research Square / Society (DOI: 10.21203/rs.3.rs-7171868)

2. Title: A graph placement methodology for fast chip design

- Authors: A. Mirhoseini, A. Goldie, M. Yazdanbakhsh, et al.

- Year: 2021

- Source: Nature (Vol. 594, pp. 207–212)

3. Title: OpenROAD: A unified, open-source physical design suite (Foundational Whitepaper)

- Authors: T. Ajayi, et al.

- Year: 2019

- Source: IEEE/ACM International Conference on Computer-Aided Design (ICCAD)

4. Title: Efficient Chip Macro Placement via Discrete Soft Actor-Critic with Enhanced Channel Attention

- Authors: L. Fan, C. Ding, J. Deng, H. Liu, and G. Feng

- Year: 2026 (March)

- Source: IEICE Transactions on Electronics

B. Dataset(s):
Dataset 1: Open-Source RISC-V Cores (e.g., PicoRV32)

- Source: OpenROAD Project / GitHub.

- Description: Industry-standard Verilog RTL designs. These serve as the raw, baseline inputs for our physical design pipeline, providing real-world connectivity graphs and macro dimensions.

Dataset 2: OpenROAD Automated Harvesting Dataset

- Source: Custom Python/TCL scripting pipeline interfacing with OpenROAD on the aforementioned RISC-V cores.

- Description: Because supervised ML models require thousands of valid ground-truth layouts, we will automate OpenROAD to run placement on Dataset 1 under randomized constraints (varying aspect ratios, utilization targets, and blockages). This will extract 10,000+ highly optimized .def layouts to train our Supervised ML Predictor.

# Timeline

- Week 1–2: Build the TCL/Python scripting pipeline to automate OpenROAD. Generate the thousands of highly optimized .def datasets required for supervised learning.

- Week 3–4: Train the Python ML Predictor on the extracted datasets. Write the Python parser to convert .def files into .hex format for the hardware initialization.

- Week 5–6: Design and simulate the parallel Verilog RTL SA Legalizer. Implement testbench file I/O for coordinate extraction and lock down the internal bit-widths for calculations to prevent integer overflow.

- Week 7–8: Build the Python subprocess orchestration script. Verify the native OpenROAD GUI rendering of the hardware-legalized layouts and ensure STA tools can successfully parse the modified DEF structure.

- Week 9+: Execute final benchmarking. To avoid software simulation bias, calculate theoretical hardware latency using the equation: . Compare this against the pure software baseline, run final OpenROAD routing, and prepare defense documentation.

# Commitment Declaration

We declare that:
- We will actively participate.
- We will attend all meetings and deadlines.
- Inactivity may result in removal from internship.
- We take full responsibility for project completion.

Mentee’s Signatures:

K Sahana

11/04/2026

Ratik Agrawal
11/04/2026

Neha Ramiah
11/04/2026

Approved By: